George E Bambara, MS, DMD, FACD, FICD

www.AttachmentDentistry.com

Tel. 917-678-2348

Email:  gbambara@AttachmentDentistry.com

The Bar Overdenture

Bar Types:

Bar Joints- Straight line- Resilient, allows prosthesis

to rotate around bar- tissue supported

Bar Unit- Bent bar, horseshoe shaped, straight bar

distalized- Rigid, does not allow prosthesis to

rotate around bar- tissue or tooth/implant

supported

Bar Shapes:

Round, Hader, Ovoid, Square, Rectangular,

Double parallel

Bar Designs:

Cast bar, Precision milled bar, Milled bar with cast

superstructure, Spark Erosion bar, Andrews bar and

sleeve, Branson bar etc.

Bar Advantages:

Directs forces down long axis of teeth or implants,

splints teeth or implants, greater stability and retention,

resists lateral forces, non-parallel abutments

Bar Disadvantages:

Bulkiness,	space requirements, increased costs

Anterior placed roots or implants connected to bar will depend more on posterior soft tissue support

Anterior and posterior placed roots or implants connected to a bar will need less tissue support